The table below is listed in Character Collating Sequence.

| BCD Character | Print | Card | BCD w/oM | Operation | Definition & Notes |
| --- | --- | --- | --- | --- | --- |
| Blank |  |  | C |  |  |
| . | . | 12-3-8 | BA821 | Halt |  |
| ¤ | ¤ | 12-4-8 | CBA84 | Clear Word Mark | Lozenge |
| [ |  | 12-5-8 | BA841 |  |  |
| < |  | 12-6-8 | BA842 |  | Less Than |
|  |  | 12-7-8 | CBA8421 |  | Group Mark |
| & | & | 12 | CBA |  |  |
| $ | $ | 11-3-8 | CB821 |  |  |
| * | * | 11-4-8 | B84 |  |  |
| ] |  | 11-5-8 | CB841 |  |  |
| ; |  | 11-6-8 | CB842 |  |  |
| Δ |  | 11-7-8 | B8421 |  | Delta (Mode Change) |
| - | - | 11 | B |  |  |
| / | / | 0-1 | CA1 | Clear Storage |  |
| , | , | 0-3-8 | CA821 | Set Word Mark |  |
| % | % | 0-4-8 | A84 | Divide | Optional special feature. |
| ˠ |  | 0-5-8 | CA841 |  | Word Separator |
| \ |  | 0-6-8 | CA842 |  | Left Oblique |
| ⧻ |  | 0-7-8 | A8421 |  | Tape Segment Mark |
| ƀ | ‡ | N/A 0 | A |  | Cannot be read from card. Punches as zero. Blank with "even-parity" on tape. |
| # | # | 3-8 | 821 | Modify Address | Optional (requires more than 4000 characters of memory) |
| @ | @ | 4-8 | C84 | Multiply | Optional special feature. |
| : |  | 5-8 | 841 |  |  |
| > |  | 6-8 | 842 |  | Greater Than |
| √ˉ |  | 7-8 | C8421 |  | Tape Mark |
| ? | & | 12-0 | CBA82 | Zero and Add | Plus Zero |
| A | A | 12-1 | BA1 | Add |  |
| B | B | 12-2 | BA2 | Branch |  |
| C | C | 12-3 | CBA21 | Compare |  |
| D | D | 12-4 | BA4 | Move Numerical | (Bits) |
| E | E | 12-5 | CBA41 | Move Characters and Edit |  |
| F | F | 12-6 | CBA42 | Control Carriage | (Printer) |
| G | G | 12-7 | BA421 |  |  |
| H | H | 12-8 | BA8 | Store B-Address Register | Optional special feature. |
| I | I | 12-9 | CBA81 |  |  |
| ! | - | 11-0 | B82 | Zero and Subtract | Minus Zero |
| J | J | 11-1 | CB1 |  |  |
| K | K | 11-2 | CB2 | Select Stacker | (Card) |
| L | L | 11-3 | B21 | Load Characters to Word Mark |  |
| M | M | 11-4 | CB4 | Move Characters to Word Mark |  |
| N | N | 11-5 | B41 | No Operation |  |
| O | O | 11-6 | B42 |  |  |
| P | P | 11-7 | CB421 | Move Characters to Record or Group Mark | Optional special feature. |
| Q | Q | 11-8 | CB8 | Store A-Address Register | Optional special feature. |
| R | R | 11-9 | B81 |  |  |
| ‡ | ‡ | 0-2-8 | A82 |  | Record Mark |
| S | S | 0-2 | CA2 | Subtract |  |
| T | T | 0-3 | A21 |  |  |
| U | U | 0-4 | CA4 | Control Unit | (Tape) |
| V | V | 0-5 | A41 | Branch if Word Mark and/or Zone |  |
| W | W | 0-6 | A42 | Branch if Bit Equal | Optional special feature. |
| X | X | 0-7 | CA421 | Move and Insert Zeros | Optional special feature. |
| Y | Y | 0-8 | CA8 | Move Zone | (Bits) |
| Z | Z | 0-9 | A81 | Move Characters and Suppress Zeros |  |
| 0 | 0 | 0 | C82 |  |  |
| 1 | 1 | 1 | 1 | Read a Card |  |
| 2 | 2 | 2 | 2 | Print a Line |  |
| 3 | 3 | 3 | C21 | Write and Read |  |
| 4 | 4 | 4 | 4 | Punch a Card |  |
| 5 | 5 | 5 | C41 | Read and Punch |  |
| 6 | 6 | 6 | C42 | Write and Punch |  |
| 7 | 7 | 7 | 421 | Write, Read, and Punch |  |
| 8 | 8 | 8 | 8 | Start Read Feed | Optional special feature. |
| 9 | 9 | 9 | C81 | Start Punch Feed | Optional special feature. |