JOB  FIRST PROGRAM

CTL  6611  *6=16,000C;6=16,000T;1=OBJDECK;,1=MODADD

ORG  400

*

MESS      DCW  @HI BILL @

BEGIN     MCW  MESS,210

W

H    BEGIN

NOP

END  BEGIN

CLEAR STORAGE 1     ,008015,022026,030037,044,049,053053N000000N00001026                                         1

CLEAR STORAGE 2     L068116,105106,110117B101/I9I#071029C029056B026/B001/0991,001/001117I0?                      2

BOOTSTRAP           ,008015,022029,036040,047054,061068,072/061039              ,0010011040                      3

FIRST PROGRAM                                                                  PAGE    1

0 SEQ PG LIN  LABEL  OP    OPERANDS                                            SFX CT  LOCN  INSTRUCTION TYPE  CARD

101                JOB   FIRST PROGRAM

102                CTL   6611  *6=16,000C;6=16,000T;1=OBJDECK;,1=MODADD

103                ORG   400                                                                0400

104         *

105         MESS   DCW   @HI BILL @                                               8  0407                       4

106         BEGIN  MCW   MESS,210                                                 7  0408  M 407 210            4

107                W                                                              1  0415  2                    4

108                H     BEGIN                                                    4  0416  . 408                4

109                NOP                                                            1  0420  N                    4

110                END   BEGIN                                                             / 408 080

1                          FIRST PROGRAM                                                                  PAGE    2

0SYMBOL  ADDRESS  SYMBOL  ADDRESS  SYMBOL  ADDRESS  SYMBOL  ADDRESS  SYMBOL  ADDRESS  SYMBOL  ADDRESS  SYMBOL  ADDRESS

BEGIN     408    MESS      407

00000000011111111112222222222333333333344444444445555555555666666666677777777778

12345678901234567890123456789012345678901234567890123456789012345678901234567890

,008015,022026,030037,044,049,053053N000000N00001026                   0001

L068116,105106,110117B101/I9I#071029C029056B026/B001/0991,001/001117I0?0002

,008015,022029,036040,047054,061068,072/061039              ,00100110400003

HI BILL M4072102.408N                  L021420,408415,416420,04004010400004

/408080                         0005

Details

- The 1401 server program will be ORG at 14 000

- This leaves the lower 12 000 locations for the loaded program

- The client PC program will send a command over $LOAD$ to the 1401 server program

- The 1401 server program will initialize a card counter to zero

- The 1401 server program will read a record from the PC client program into location 001 like a normal card read

- Increment the card counter

- If the first column is not a column,

  - Print the bad card on the printer followed by “Col 1 of card 1 not a comma”

  - Halt

  - Go to start of program

- Read a second record from the PC client program into location 001 like a normal card read

- Increment the card counter

- If the first column is not an L,

  - Print the bad card on the printer followed by “Col 1 of card 2 not a L”

  - Halt

  - Go to start of program

- Clear memory from 001 to 14 000

- Read a record from the PC client program into location 001 like a normal card read

  - This is address aaa

- Increment the card counter

- Move “Baaa to 071. (aaa)  this replaced the read a card and branch 040 in the record

- Turn over control to the record B 001

  - When the record finishes, it will return control to (aaa)

- The last record will pass control to the loaded program never to return.

- The first statement in The 1401 server program will be DSA START

  - This will give a fixed location to be able to restart the program without having to reload it from a deck

- The PC Client program will;

  - Ask the user what action is requested

  - If it is LOAD the user will select the file

  - A $LOAD$ is sent to the IBM 1401

  - A record is read from the selected file

  - The record is sent to the IBM 1401

  - If location 40 is not a “/” go to step d

  - Else go to the top menu.